www.ruris.ro.

TILLER DAC 185 PRO

| Engine | General Engine |
| --- | --- |
| Operating cycle | 4 times |
| Net power | 7.5 hp |
| Maximum power | 8.5 hp |
| Cylindrical capacity | 208 cc |
| Ignition system | Electronic |
| Emergency stop lever | Standard equipment |
| Starting | Electric start, 18A battery |
| Lighting system | - |
| Gear shifter | Comfort-fast |
| Transmission | Pinions in oil bath |
| Transmission housing | Nodular Cast Iron |
| Cable drive | Mechanics |
| speeds | 2 forward + 1 backward |
| Working width | 560-930 mm |
| Working depth | Max 340 mm |
| Combustible | Unleaded gasoline |
| Tank capacity | 3 l |
| Average fuel consumption | 1l/h |
| Engine oil bath | 0.6 l |
| Transmission oil bath | 1.8 l |
| Average worked surface | 600-1000 sq/h |
| Tire wheel | 19*7-8 |
| Net weight with accessories | 80 kg |
| Handle vibrations | left 4.84m/s² - right 5.82m/s² |
| Product warranty | 24 months |

www.ruris.ro.

producãtor motoutilaje   www.ruris.ro.